# FUEL INJECTION DYNAMICS

## 1. INTRODUCTION

Fuel injection dynamics describe the physical and thermodynamic behavior of fuel as it is delivered from an injector into a combustion chamber. In the PNG (Pure Natural Gas) Energy System, this process defines the initial boundary condition for downstream atomization, mixing, ignition, and combustion efficiency.

This model formalizes injection as a time-dependent fluid–energy control system rather than a static flow event.

### Engineering objectives

- Controlled fuel mass delivery

- Optimized spray formation

- Improved air–fuel mixing

- Reduced incomplete combustion losses

---

## 2. GOVERNING PRINCIPLES OF FUEL INJECTION

### 2.1 Nozzle Flow Dynamics

Fuel discharge is governed by pressure differential across a calibrated orifice.

**Equation:**

Q = C_d A √(2ΔP / ρ)

**Variables:**

- Q = volumetric flow rate (m³/s)

- C_d = discharge coefficient

- A = nozzle area (m²)

- ΔP = pressure difference (Pa)

- ρ = fuel density (kg/m³)

**Interpretation:**

- Higher ΔP → higher jet velocity → better atomization

- Smaller A → higher velocity but reduced mass flow

---

### 2.2 Time-Dependent Injection Dynamics

Real injection systems are transient:

Q(t) = C_d A √(2ΔP(t) / ρ)

**Drivers:**

- Injector response time

- Pressure wave dynamics

- Valve actuation delay

Injection behaves as a **dynamic control system**, not a steady flow.

---

### 2.3 Spray Formation Physics

Jet breakup occurs through instability mechanisms:

- Primary breakup → liquid core disintegration

- Secondary breakup → droplet fragmentation

**Key dimensionless effects:**

- Reynolds number → turbulence intensity

- Weber number → surface tension vs aerodynamic force

**Outputs:**

- Spray penetration length

- Cone angle

- Droplet size distribution

—

## Figure 2.1 – Fuel Injection Flow and Spray Formation

### Description

This figure illustrates the transition of fuel from a high-pressure liquid jet at the injector nozzle into a dispersed multiphase spray structure inside the combustion chamber.

### Components shown

- Injector nozzle under high-pressure supply (ΔP)

- High-velocity fuel jet exiting orifice

- Primary breakup zone (continuous jet → ligaments)

- Secondary breakup zone (droplet formation)

- Spray cone development

- Atomization field inside the combustion chamber

### Engineering significance

This figure represents the fundamental physical transition in fuel injection dynamics, where:

- Pressure energy is converted into kinetic jet energy

- Jet instability leads to atomization

- Spray geometry determines air–fuel mixing efficiency

### System role in PNG Energy Architecture

Injection → Spray Formation → Air–Fuel Mixing → Ignition → Combustion → Energy Output

## 3. INJECTION SYSTEM CONTROL PARAMETERS

### 3.1 Injection Pressure (ΔP)

- Higher pressure → finer droplets

- Higher pressure → better mixing

- Higher pressure → stable combustion

---

### 3.2 Nozzle Geometry

- Orifice diameter

- L/D ratio

- Hole orientation

Controls spray angle and mixing zone formation.

---

### 3.3 Fuel Properties

- Density (ρ)

- Viscosity (μ)

Effects:

- Higher viscosity → larger droplets

- Higher density → higher jet momentum

---

### 3.4 Thermal Conditions

- High temperature → improved vaporization

- Low temperature → liquid persistence

---

### 3.5 Injection Timing (SOI / DOI / EOI)

- SOI = Start of Injection

- DOI = Duration of Injection

- EOI = End of Injection

Controls:

- Combustion phasing

- Ignition delay

- Peak pressure timing

---

## 4. NUMERICAL ANALYSIS

Given:

- C_d = 0.85

- A = 1.5 × 10⁻⁶ m²

- ΔP = 3 × 10⁶ Pa

- ρ = 750 kg/m³

**Calculation:**

Q = C_d A √(2ΔP / ρ)

Q ≈ 1.14 × 10⁻⁴ m³/s

### 4.1 Interpretation

- Q ≈ 0.114 L/s

- High-velocity injection regime

- Supports fine atomization

---

## 5. COMBUSTION SYSTEM ARCHITECTURE

Injection → Spray Formation → Air–Fuel Mixing → Ignition Kernel Formation → Flame Propagation → Energy Output

### Functional mapping

- Injection → momentum input

- Spray → spatial distribution

- Mixing → equivalence ratio control

- Ignition → combustion start

- Flame → energy release rate

---

## 6. ENGINEERING APPLICATIONS

- Internal combustion engines (SI / CI)

- Gas energy systems

- Hybrid PNG catalytic systems

- Injection research platforms

---

## 7. OPTIMIZATION FRAMEWORK

### Variables

- ΔP (pressure)

- A (geometry)

- SOI / DOI / EOI

- Fuel properties

### Objective

Maximize combustion efficiency (ηc)

### Constraints

- Stable ignition

- Low emissions

- Controlled pressure rise

---

## 8. SYSTEM INTEGRATION (PNG ARCHITECTURE)

- Fuel Atomization Module

- Catalyst Process Module

- Combustion Output Module

Role:

- Defines boundary condition

- Controls reaction efficiency

- Stabilizes system output

---

## 9. CONCLUSION

Fuel injection dynamics define the foundational control layer of the PNG Energy System. Performance is governed by pressure, geometry, timing, and fuel properties.